# AuraBridge Pi 2.2 Project Overview

## 1. Project Summary

AuraBridge Pi 2.2 is a Raspberry Pi 4 based multi-protocol wireless audio receiver for a Harman Kardon Aura Studio 3 speaker.

The Raspberry Pi receives audio through AirPlay 2, Spotify Connect, Bluetooth A2DP, and optional DLNA / UPnP. It routes audio through PipeWire and WirePlumber to a controlled audio path, then to a FiiO KA11 Type-C USB DAC / headphone amplifier. The KA11 outputs analog audio through a 3.5 mm AUX cable into the Aura Studio 3.

The project does not modify the speaker internally. The Raspberry Pi works as an external audio bridge.

Main signal path:

User devices
  -> AirPlay 2 / Spotify Connect / Bluetooth A2DP / optional DLNA
  -> Raspberry Pi 4
  -> PipeWire + WirePlumber
  -> AuraBridge Safe Sink, preferred controlled sink
  -> PipeWire limiter or fixed-gain stage, if verified
  -> FiiO KA11 Type-C USB DAC
  -> 3.5 mm AUX
  -> Harman Kardon Aura Studio 3

## 2. Version 2.2 Design Changes

Version 2.2 makes the following corrections:

- volume-guard-loop.sh is not a real-time safety mechanism.

- Polling-based volume correction is only for recovery, audit, and diagnostics.

- Real-time audio safety must be implemented inside the PipeWire / WirePlumber audio graph, or risky clients must remain disabled.

- DLNA is blocked until a PipeWire-level limiter, virtual safe sink, fixed-gain stage, or equivalent hard cap is verified.

- The FiiO KA11 physical sink should not be exposed as the default sink for normal clients if a Safe Sink is implemented.

- WirePlumber policy must be written according to the actual installed WirePlumber version.

- Do not blindly copy latest WirePlumber examples.

- Shairport Sync should use the PulseAudio backend through pipewire-pulse for the MVP.

- The MVP controller should be Bash scripts plus systemd services/timers, not FastAPI or Node.js.

## 3. Main Goal

Build a stable, headless, practical multi-protocol audio bridge for daily home use.

The system must support:

AirPlay 2 from iPhone, iPad, and Mac
Spotify Connect through librespot
Bluetooth A2DP for Android, Xiaomi, Samsung, and PC devices
Optional DLNA / UPnP only after real-time safety is verified
FiiO KA11 USB DAC output
Safe initial volume
Real-time audio safety layer before risky clients
Service health checks
Useful diagnostic logs
systemd startup and recovery

## 4. Non-Goals

Do not attempt:

Native Xiaomi MiPlay receiver support
Samsung SmartThings / Tap Sound / Music Share emulation
Full Google Cast / Chromecast receiver
Internal modification of the Aura Studio 3
Raspberry Pi onboard 3.5 mm output as the final output
Passive Type-C to 3.5 mm analog passthrough adapters
Apple Home app stability as a required feature
Pure ALSA direct output as the multi-protocol default
FastAPI or Node.js controller in the MVP
Polling-based Bash volume guard as a real-time safety mechanism
DLNA before real-time audio safety is verified

## 5. Required Hardware

Raspberry Pi 4 Model B
microSD card, 32 GB or larger recommended
Official or high quality 5V 3A USB-C power supply
USB-A male to Type-C female adapter
FiiO KA11 Type-C USB DAC / headphone amplifier
3.5 mm AUX cable
Harman Kardon Aura Studio 3

Optional:

Ethernet cable
Raspberry Pi case
Heatsink or fan
Powered USB hub
I2C OLED display
Physical button for Bluetooth pairing or safe-volume reset

## 6. FiiO KA11 Requirements

The FiiO KA11 Type-C is the project DAC.

The agent must verify that it is detected by Linux as a USB audio device.

Required checks:

lsusb
aplay -l
amixer -c <card_id> scontrols
alsamixer -c <card_id>
pipewire --version
wireplumber --version
wpctl status
pactl list sinks short

Expected outcome:

KA11 appears in lsusb.
KA11 appears as an ALSA sound card.
KA11 appears as a PipeWire sink.
WirePlumber version is recorded.
Hardware mixer behavior is documented.
The default output path can route audio to KA11.
Test audio can play through KA11 into the Aura Studio 3.

If the KA11 does not appear as a USB audio device, stop the setup and report the error.

## 7. Important KA11 Safety Warning

The KA11 is a USB DAC / headphone amplifier. It is not a fixed-level line-out device.

When connected to an active speaker AUX input, high output volume can cause very loud sound.

The system must apply safe volume before any real playback test.

Initial safety values:

PipeWire default sink volume: 20% to 30%
Maximum normal testing volume: 45%
Aura Studio 3 physical volume: low
Phone / source device volume: low
DLNA: disabled until real-time safety is verified

Required command for initial safe volume:

wpctl set-volume @DEFAULT_AUDIO_SINK@ 0.30
wpctl set-mute @DEFAULT_AUDIO_SINK@ 0

Important:

This command is only initialization.
It is not a real-time limiter.
It does not protect against instant volume spikes from untrusted clients.

## 8. Audio Architecture

The system must use PipeWire and WirePlumber as the main audio routing layer.

Recommended architecture:

Shairport Sync, AirPlay 2
librespot, Spotify Connect
BlueZ + PipeWire Bluetooth, Bluetooth A2DP
Optional DLNA renderer, blocked until safe
        |
        v
pipewire-pulse / PipeWire
        |
        v
PipeWire media graph
        |
        v
WirePlumber policy manager
        |
        v
AuraBridge Safe Sink, preferred
        |
        v
Limiter or fixed-gain stage, if verified
        |
        v
FiiO KA11 USB DAC sink
        |
        v
3.5 mm AUX output
        |
        v
Aura Studio 3

Do not route all services directly to ALSA hardware devices such as:

hw:1,0
plughw:1,0

Direct ALSA output can cause device locking problems in a multi-protocol system.

## 9. Real-Time Audio Safety Requirement

This is the most important 2.2 correction.

Do not treat scripts/volume-guard-loop.sh as a real-time safety mechanism.

It is only:

Recovery
Audit
Diagnostics
Post-failure correction

It is not:

Limiter
Hard cap
Speaker protection
Real-time safety layer

Why:

If a client sends SetVolume=100% and the script checks every 5 seconds,
the speaker can still play at full volume for several seconds.
That is unacceptable.

Required design direction:

Create an AuraBridge Safe Sink whenever possible.
Route normal clients to the Safe Sink, not directly to KA11.
Route the Safe Sink through a PipeWire filter-chain limiter or fixed-gain stage.
Only then route audio to the KA11 physical sink.

If a real-time safety layer cannot be verified:

Keep DLNA disabled.
Keep untrusted clients disabled.
Do not claim the system is protected by volume-guard-loop.sh.

## 10. AuraBridge Safe Sink Design

Preferred path:

Normal clients
  -> AuraBridge Safe Sink
  -> limiter or fixed-gain stage
  -> FiiO KA11 physical sink
  -> AUX
  -> Aura Studio 3

The KA11 physical sink should not be the default sink for normal clients if the Safe Sink is implemented.

The agent must not assume that a limiter plugin exists.

Before implementing the Safe Sink, inspect the system:

pipewire --version
wireplumber --version
pw-cli ls Node
wpctl status
ls /usr/lib/*/ladspa 2>/dev/null || true
ls /usr/lib/*/lv2 2>/dev/null || true

Safe Sink acceptance:

Clients can output to AuraBridge Safe Sink.
Audio reaches KA11 through a controlled path.
KA11 physical sink is not the normal default sink.
Output gain is safe.
100% client volume does not create dangerous analog output.
If this cannot be verified, DLNA remains disabled.

## 11. Shairport Sync AirPlay 2 Design

Use:

NQPTP
Shairport Sync
PulseAudio backend
pipewire-pulse
PipeWire
FiiO KA11

MVP output path:

Shairport Sync
  -> PulseAudio backend
  -> pipewire-pulse
  -> PipeWire
  -> AuraBridge Safe Sink, if implemented
  -> FiiO KA11

Do not use the native PipeWire backend as the MVP default.

Before building, inspect available configure options:

./configure --help | grep -i pulse
./configure --help | grep -i airplay

Preferred configure idea:

./configure --sysconfdir=/etc \
  --with-pa \
  --with-soxr \
  --with-avahi \
  --with-ssl=openssl \
  --with-systemd-startup \
  --with-airplay-2

If --with-pa is not the correct flag, use the PulseAudio backend flag shown by ./configure --help.

Target AirPlay name:

Aura Studio 3 AirPlay

AirPlay acceptance:

NQPTP service is active.
Shairport Sync service is active.
pipewire-pulse is active.
iPhone or Mac sees "Aura Studio 3 AirPlay".
Audio plays through KA11 and Aura Studio 3.
PipeWire shows the stream.
No ALSA device locking conflict occurs.
Output level is safe.

## 12. Spotify Connect Design

Use librespot.

Preferred path:

librespot
  -> PulseAudio-compatible output
  -> pipewire-pulse
  -> PipeWire
  -> AuraBridge Safe Sink, if implemented
  -> FiiO KA11

Target Spotify name:

Aura Studio 3 Spotify

Acceptance:

Spotify app sees the device.
Playback works.
No ALSA device locking conflict with AirPlay.
Output level is safe.

## 13. Bluetooth A2DP Design

Use:

BlueZ
PipeWire Bluetooth support
WirePlumber BlueZ monitor
FiiO KA11 sink

Target Bluetooth name:

Aura Studio 3 BT

Bluetooth must not be permanently discoverable.

Required script:

scripts/bt-pairing-window.sh

Basic behavior:

bluetoothctl discoverable on
sleep 120
bluetoothctl discoverable off

Bluetooth routing must be tested. Do not assume the default WirePlumber policy will be ideal.

Required Bluetooth routing spike:

Record PipeWire version.
Record WirePlumber version.
Identify whether WirePlumber is 0.4.x or 0.5.x+.
Use version-matched WirePlumber documentation only.
Pair an Android phone.
Check wpctl status before and after connection.
Check pactl list sink-inputs.
Play AirPlay, then connect Bluetooth.
Play Spotify, then connect Bluetooth.
Document whether Bluetooth hijacks routing.
If hijack occurs, add version-specific mitigation.

If Bluetooth is too disruptive, the MVP may keep Bluetooth disabled by default and enable it manually only when needed.

## 14. WirePlumber Version Policy

This is a hard requirement.

Before writing any WirePlumber policy, run:

wireplumber --version
wpctl status
pw-cli info all

Then follow this rule:

If WirePlumber is 0.4.x:
  Use WirePlumber 0.4 Lua-style configuration and 0.4-era documentation.

If WirePlumber is 0.5.x or newer:
  Use the newer SPA-JSON / JSON-style configuration model.

Do not blindly copy latest WirePlumber examples.
Do not assume a 0.5.x config works on 0.4.x.
Do not assume a 0.4.x Lua rule works on 0.5.x.

Any WirePlumber configuration change must document:

Installed WirePlumber version
Config directory used
Config syntax used
Files modified
Reason for change
Rollback instructions

## 15. DLNA / UPnP Design

DLNA is blocked by default.

Do not enable DLNA until real-time audio safety is verified.

Possible tools:

gmrender-resurrect
Rygel

DLNA must not rely on volume-guard-loop.sh.

DLNA unlock requirements:

AuraBridge Safe Sink exists, or equivalent protected route exists.
KA11 physical sink is not directly exposed as default sink.
PipeWire-level limiter, fixed-gain filter, or hard cap is verified.
100% client-side volume command does not create dangerous analog output.
Test is performed with Aura Studio 3 physical volume low.
Quick disable script exists.
Exact renderer and client behavior is documented.

If these requirements are not met:

Keep DLNA disabled.
Do not install DLNA as an enabled startup service.
Do not include DLNA in the MVP acceptance criteria.

## 16. Controller Design

Do not use FastAPI or Node.js in the MVP.

Use:

Bash scripts
systemd services
systemd timers
SSH
journalctl

Possible future UI:

Tiny Python http.server
OLED status display
Physical pairing button
Physical safe-volume reset button

## 17. Required Repository Structure

AuraBridge-Pi/
  README.md
  WHITEPAPER_2_2.md
  PROJECT_OVERVIEW_2_2.md
  TROUBLESHOOTING.md

  scripts/
    setup-base.sh
    setup-pipewire.sh
    check-ka11.sh
    safe-volume.sh
    volume-guard-loop.sh
    install-airplay2.sh
    install-spotify.sh
    setup-bluetooth.sh
    bt-pairing-window.sh
    bluetooth-routing-spike.sh
    wireplumber-version-check.sh
    setup-safe-sink.sh
    test-safe-sink.sh
    install-dlna.sh
    status.sh
    logs.sh

  systemd/
    aurabridge-volume-guard.service
    aurabridge-volume-guard.timer
    aurabridge-health.service
    librespot.service
    gmrender.service

  docs/
    hardware.md
    ka11-validation.md
    audio-routing.md
    volume-safety.md
    safe-sink.md
    bluetooth-policy.md
    wireplumber-versioning.md
    airplay2.md
    spotify.md
    dlna.md

## 18. Required Scripts

### check-ka11.sh

Must print:

USB devices
ALSA sound cards
ALSA mixer controls for KA11
PipeWire version
WirePlumber version
PipeWire status
Available sinks
Default sink

Must fail clearly if the KA11 is not detected.

### safe-volume.sh

Minimum behavior:

wpctl set-volume @DEFAULT_AUDIO_SINK@ 0.30
wpctl set-mute @DEFAULT_AUDIO_SINK@ 0

Important:

This is only initialization.
It is not a real-time safety mechanism.

### volume-guard-loop.sh

Must periodically check the default sink volume.

If volume exceeds the configured maximum, clamp it back to a safe value.

Suggested values:

safe volume: 0.30
maximum allowed volume during MVP: 0.45
check interval: 5 to 10 seconds

Important:

This script is only for recovery and diagnostics.
It must not be documented as speaker protection.
It must not be used to justify enabling DLNA.

### wireplumber-version-check.sh

Must print:

WirePlumber version
PipeWire version
Config directories
Likely config model, 0.4 Lua or 0.5+ SPA-JSON
Warning not to copy mismatched docs

### setup-safe-sink.sh

Must attempt to create or document a protected output path.

If it cannot create a real-time safe path, it must print:

Real-time audio safety layer not verified.
DLNA must remain disabled.

### status.sh

Must print:

AuraBridge Pi Status
Hostname
IP address
KA11 detection status
PipeWire version
WirePlumber version
Default sink
Default sink volume
Safe Sink status
AirPlay service status
NQPTP service status
Spotify service status
Bluetooth status
DLNA status
Recent errors

### logs.sh

Must collect:

journalctl -u shairport-sync -n 100 --no-pager
journalctl -u nqptp -n 100 --no-pager
journalctl -u bluetooth -n 100 --no-pager
journalctl -u librespot -n 100 --no-pager
journalctl --user -u pipewire -n 100 --no-pager
journalctl --user -u wireplumber -n 100 --no-pager
dmesg | grep -i usb

## 19. Development Phases

### Phase 0: Preparation

Prepare Raspberry Pi OS Lite 64-bit.
Prepare microSD card.
Prepare Pi 4 power supply.
Prepare USB-A to Type-C female adapter.
Prepare FiiO KA11 Type-C.
Prepare AUX cable.
Prepare Ethernet cable if possible.
Prepare this project overview and prompts.

### Phase 1: Base OS, PipeWire, WirePlumber, and KA11 Validation

Goal:

Boot Pi.
Install PipeWire and WirePlumber.
Detect KA11.
Record PipeWire and WirePlumber versions.
Document KA11 hardware mixer behavior.
Set initial safe volume.
Play test audio through Aura Studio 3.

Acceptance:

Pi is reachable over SSH.
KA11 appears in lsusb.
KA11 appears in aplay -l.
KA11 appears in wpctl status.
PipeWire version is recorded.
WirePlumber version is recorded.
Hardware mixer behavior is documented.
Safe volume works.
Test audio plays at low volume.

### Phase 2: AirPlay 2

Goal:

Install NQPTP.
Build and install Shairport Sync with AirPlay 2 support and PulseAudio backend.
Route AirPlay through pipewire-pulse to KA11 or Safe Sink.

Acceptance:

AirPlay target appears.
Playback works.
PipeWire shows stream.
No ALSA device locking conflict occurs.
Output level is safe.

### Phase 3: Spotify Connect

Goal:

Install librespot as a systemd service.
Route Spotify through pipewire-pulse to KA11 or Safe Sink.

Acceptance:

Spotify app sees device.
Playback works.
No AirPlay conflict.
Output level is safe.

### Phase 4: Bluetooth A2DP

Goal:

Enable Bluetooth A2DP receiving with controlled pairing.
Test routing behavior.
Prevent or document auto-connect interruptions.

Acceptance:

Android phone can pair.
Bluetooth playback works.
Pairing window closes automatically.
Routing behavior is documented.
WirePlumber version-specific mitigation exists if Bluetooth hijacks routing.

### Phase 5: Real-Time Audio Safety / Safe Sink

Goal:

Implement or verify a real-time audio safety layer.

Acceptance:

AuraBridge Safe Sink exists, or equivalent protected route exists.
KA11 physical sink is not the default normal client sink.
Limiter, fixed-gain stage, or hard cap is verified.
100% client-side volume does not create dangerous analog output.
If this cannot be verified, DLNA remains disabled.

### Phase 6: Optional DLNA

Goal:

Add DLNA only after real-time safety testing.
Keep it disabled by default.

Acceptance:

DLNA can be enabled manually.
Playback works.
No unsafe volume spike occurs.
DLNA can be quickly disabled.
Behavior is documented.

### Phase 7: Optional Convenience Layer

Goal:

Add lightweight local control only after the audio system is stable.

Allowed:

Bash scripts
systemd timers
Tiny Python http.server
OLED status screen
Physical button

Avoid in MVP:

FastAPI
Node.js Express
Heavy frontend framework

## 20. Core MVP Acceptance Criteria

[ ] Raspberry Pi 4 boots headless.
[ ] SSH works.
[ ] FiiO KA11 is detected.
[ ] PipeWire sees KA11.
[ ] PipeWire version is recorded.
[ ] WirePlumber version is recorded.
[ ] KA11 hardware mixer behavior is documented.
[ ] Safe initial volume is applied.
[ ] AirPlay 2 works.
[ ] Spotify Connect works.
[ ] Services survive reboot.
[ ] status.sh reports meaningful state.
[ ] logs.sh collects useful diagnostics.

## 21. MVP Plus Acceptance Criteria

[ ] Bluetooth A2DP works.
[ ] Bluetooth pairing is controlled.
[ ] Bluetooth is not permanently discoverable.
[ ] Bluetooth auto-connect behavior is documented.
[ ] Bluetooth does not permanently break AirPlay or Spotify.
[ ] WirePlumber policy is version-specific if used.

## 22. Real-Time Safety Acceptance Criteria

[ ] volume-guard-loop.sh is not treated as speaker protection.
[ ] Safe Sink or equivalent protected path is tested.
[ ] KA11 physical sink is not the normal default sink if Safe Sink is implemented.
[ ] Limiter or fixed-gain route is verified if available.
[ ] 100% client volume does not cause dangerous analog output.
[ ] DLNA remains disabled if real-time safety is not proven.

## 23. Optional DLNA Acceptance Criteria

[ ] DLNA is disabled by default.
[ ] DLNA can be manually enabled.
[ ] DLNA playback works.
[ ] DLNA does not force unsafe volume.
[ ] DLNA can be disabled quickly.

## 24. Major Risks and Mitigations

| Risk | Severity | Mitigation |
| --- | --- | --- |
| ALSA device locking | High | Use PipeWire / pipewire-pulse, avoid direct hw: output |
| DLNA volume spike | High | Keep DLNA blocked until Safe Sink / limiter is verified |
| Bash volume guard race condition | High | Treat polling as recovery only |
| Bluetooth auto-connect interruption | Medium | Controlled pairing, routing spike, version-specific WirePlumber policy |
| WirePlumber version mismatch | High | Detect version first, use matching docs |
| KA11 output too loud | High | Low physical volume, safe initial volume, hardware mixer check |
| USB DAC not detected | High | Validate KA11 with check-ka11.sh |
| Wi-Fi instability | Medium | Prefer Ethernet or stable 5 GHz Wi-Fi |
| Apple Home app instability | Low | Best-effort only |
| Controller over-engineering | Low | Use Bash + systemd in MVP |

## 25. Recommended MVP Scope

Do not build everything at once.

Recommended first build:

Raspberry Pi OS Lite 64-bit
PipeWire
WirePlumber
FiiO KA11 validation
Safe initial volume
NQPTP
Shairport Sync AirPlay 2 through PulseAudio backend
librespot Spotify Connect
status and logs scripts

Then add:

Bluetooth A2DP with controlled pairing
Bluetooth routing spike
WirePlumber version-specific policy only if needed

Only after that add:

AuraBridge Safe Sink
PipeWire limiter or fixed-gain route
Optional DLNA after safety verification

## 26. Final Definition

AuraBridge Pi 2.2 is a Raspberry Pi 4 based multi-protocol audio bridge that turns a traditional AUX speaker into a practical network audio endpoint.

It uses PipeWire and WirePlumber as the central audio routing layer, Shairport Sync and NQPTP for AirPlay 2, librespot for Spotify Connect, BlueZ and PipeWire for Bluetooth A2DP, optional DLNA only after safety verification, and the FiiO KA11 Type-C as the dedicated USB DAC output device.

The core 2.2 rule is:

Audio safety must be real-time and inside the audio graph.
Polling scripts are not speaker protection.
DLNA stays disabled until a Safe Sink, limiter, or equivalent hard cap is verified.
WirePlumber policy must match the installed WirePlumber version.