# AuraBridge Pi 2.2 Project Whitepaper

## 基于 Raspberry Pi 4、PipeWire、WirePlumber、FiiO KA11 与实时音频安全层的多协议家庭音频桥接系统

## 1. 项目概述

AuraBridge Pi 2.2 是一个基于 Raspberry Pi 4 的多协议家庭无线音频接收系统。项目目标是把 Harman Kardon Aura Studio 3，也就是琉璃3，通过一个外置 Raspberry Pi 音频桥升级为支持 AirPlay 2、Spotify Connect、Bluetooth A2DP，以及可选 DLNA / UPnP 的现代网络音频终端。

本项目不拆解琉璃3，不修改音箱内部电路，不刷音箱固件。Raspberry Pi 4 负责无线协议接收、音频服务管理、音频路由、音量安全和系统监控。FiiO KA11 Type-C 作为指定 USB DAC / headphone amplifier，负责把 Raspberry Pi 输出的数字 USB 音频转换成模拟 3.5mm 音频信号，再通过 AUX 输入连接琉璃3。

2.2 版相比 2.1 版的核心修正是：

- volume-guard-loop.sh 不再被视为实时安全机制，只能作为 recovery、audit 和 diagnostics 工具。

- 实时音量安全必须在 PipeWire / WirePlumber 音频图内部实现，或者在无法验证之前禁用高风险协议。

- DLNA 被标记为 blocked feature，必须等 PipeWire-level limiter、virtual safe sink 或等效硬限制验证成功后才能启用。

- FiiO KA11 物理 sink 不应直接暴露为长期默认 sink，推荐通过 AuraBridge Safe Sink 进行受控输出。

- WirePlumber policy 必须基于实际安装版本编写，不能盲目复制最新版配置示例。

- Shairport Sync MVP 默认使用 PulseAudio backend，通过 pipewire-pulse 接入 PipeWire，而不是使用 native PipeWire backend。

- Controller MVP 保持 Bash scripts + systemd services/timers，不引入 FastAPI 或 Node.js。

最终目标是做出一个稳定、可维护、可扩展、适合日常使用，也适合作为 embedded Linux / Linux system engineering 项目展示的音频桥接设备。

## 2. 总体系统链路

iPhone / Mac / iPad / Android / Xiaomi / Samsung / PC
        ↓
AirPlay 2 / Spotify Connect / Bluetooth A2DP / optional DLNA
        ↓
Raspberry Pi 4
        ↓
PipeWire + WirePlumber
        ↓
AuraBridge Safe Sink, virtual controlled sink
        ↓
PipeWire filter-chain limiter or fixed-gain stage
        ↓
FiiO KA11 Type-C USB DAC
        ↓
3.5mm AUX cable
        ↓
Harman Kardon Aura Studio 3

2.2 版的核心设计原则是：

Normal clients should not directly target the FiiO KA11 physical sink.
They should target an AuraBridge Safe Sink whenever possible.
The Safe Sink should enforce gain limiting or real-time protection before audio reaches KA11.

## 3. 设计背景

琉璃3是一台声音和外观都不错的桌面蓝牙音箱，但它缺少现代网络音频能力：

- 没有原生 AirPlay 2。

- 不能稳定作为 Apple Home 认证音箱。

- 没有 Spotify Connect。

- 没有 DLNA / UPnP renderer。

- 蓝牙连接是点对点体验，多设备切换不够优雅。

- Android、小米、三星生态没有一个完全等价于 AirPlay 的统一开放协议。

本项目采用外置音频桥方案，不尝试破解或模拟小米妙播、三星 SmartThings、Google Cast 等私有生态，而是使用稳定、通用、可维护的协议组合：

Apple devices: AirPlay 2
Spotify users: Spotify Connect
Android / Xiaomi / Samsung: Bluetooth A2DP
Local network media apps: optional DLNA / UPnP, blocked until safety is verified

## 4. 项目目标

### 4.1 Core MVP 目标

第一版需要实现：

- Raspberry Pi 4 headless 启动。

- FiiO KA11 Type-C 被 Linux 识别为 USB Audio DAC。

- PipeWire 能识别 KA11 为 audio sink。

- 系统能建立受控的默认输出路径。

- AirPlay 2 可以从 iPhone / Mac 播放到琉璃3。

- Spotify Connect 可以从 Spotify app 播放到琉璃3。

- 所有核心服务通过 systemd 管理。

- 提供 status 和 logs 脚本。

- 服务重启后能自动恢复。

- 不发生明显爆音、满音量冲击或 ALSA device busy 冲突。

### 4.2 MVP Plus 目标

第二阶段加入：

- Bluetooth A2DP 接收。

- 手动开启蓝牙配对窗口。

- 避免永久 discoverable。

- 测试蓝牙自动连接是否会抢占 AirPlay / Spotify。

- 必要时加入 WirePlumber policy 或中控脚本修正。

### 4.3 Optional 目标

后续可以加入：

- DLNA / UPnP renderer。

- AuraBridge Safe Sink。

- PipeWire filter-chain limiter。

- I2C OLED 状态显示。

- 物理按钮，用于蓝牙配对或 safe-volume reset。

- source priority controller。

- 极轻量本地 status page。

## 5. 非目标

本项目不做以下内容：

- 不实现小米妙播 receiver。

- 不实现三星 SmartThings / Tap Sound / Music Share receiver。

- 不实现完整 Chromecast receiver。

- 不拆解或改造琉璃3。

- 不使用 Raspberry Pi 板载 3.5mm 作为最终输出。

- 不使用无源 Type-C 转 3.5mm 模拟直通线。

- 不把 Apple Home app 稳定加入作为核心验收标准。

- 不在多协议版本中使用纯 ALSA direct output 作为主架构。

- 不在 MVP 中引入大型 Web framework。

- 不使用 Bash polling 作为实时音频安全机制。

- 不在未验证 limiter / safe sink 前启用 DLNA。

## 6. 指定硬件架构

### 6.1 必需硬件

Raspberry Pi 4 Model B
microSD card, 32GB or above recommended
Official or high quality 5V 3A USB-C power supply
USB-A male to Type-C female adapter
FiiO KA11 Type-C USB DAC / headphone amplifier
3.5mm AUX cable
Harman Kardon Aura Studio 3

### 6.2 可选硬件

Ethernet cable
Raspberry Pi case
Heatsink or fan
Powered USB hub
I2C OLED display
Physical button

### 6.3 FiiO KA11 定位

FiiO KA11 Type-C 在本项目中被定义为：

Primary USB DAC
Headphone amplifier output stage
3.5mm analog AUX source for Aura Studio 3

注意，KA11 是 DAC / headphone amplifier，不是固定电平 line-out。它的输出能力较强，直接接入有源音箱 AUX 时必须谨慎控制音量。

因此，2.2 版增加以下安全规则：

- 首次测试时，琉璃3音量必须调低。

- Raspberry Pi / PipeWire 默认音量必须先设为 20% 到 30%。

- KA11 如果暴露硬件 mixer，需要检查并设定安全硬件音量。

- 禁止在未知状态下播放高音量测试信号。

- DLNA 不允许在未验证实时 audio safety layer 前启用。

- KA11 physical sink 不应直接作为正常客户端的长期默认 sink。

## 7. 硬件连接

推荐连接方式：

Raspberry Pi 4 USB-A port
        ↓
USB-A male to Type-C female adapter
        ↓
FiiO KA11 Type-C
        ↓
3.5mm AUX cable
        ↓
Aura Studio 3 AUX-IN

注意事项：

- 不要使用 Pi 4 板载 3.5mm 输出。

- KA11 需要被系统识别为 USB audio device。

- 如果 KA11 插上后 lsusb 没反应，需要检查转接头方向、供电、线材或系统 USB 枚举问题。

- 如果出现 USB reset、断音、爆音，尝试换 USB 口、换电源，或者使用 powered USB hub。

## 8. 系统软件架构

### 8.1 总体架构

Protocol Receiver Layer
  - Shairport Sync, AirPlay 2
  - librespot, Spotify Connect
  - BlueZ + PipeWire Bluetooth, Bluetooth A2DP
  - optional gmrender / Rygel, DLNA, blocked until safe

Audio API Layer
  - pipewire-pulse for PulseAudio-compatible clients
  - PipeWire native graph where appropriate

Policy Layer
  - WirePlumber
  - version-specific configuration only

Safety Layer
  - AuraBridge Safe Sink, preferred
  - PipeWire filter-chain limiter or fixed-gain stage, if verified
  - KA11 physical sink not exposed as default sink if possible

Output Layer
  - FiiO KA11 USB DAC sink

Analog Playback Layer
  - 3.5mm AUX to Aura Studio 3

### 8.2 为什么不用纯 ALSA

纯 ALSA direct output 适合单一 AirPlay receiver，但不适合作为多协议系统主架构。原因：

- 服务可能独占 USB DAC。

- AirPlay、Spotify、Bluetooth、DLNA 同时运行时容易发生 Device or resource busy。

- 蓝牙自动连接可能改变默认路由。

- DLNA 音量控制不可靠。

- 后续 dashboard、volume guard、source policy 更难做。

因此，2.2 版采用：

Shairport Sync / librespot / Bluetooth / optional DLNA
        ↓
pipewire-pulse or PipeWire
        ↓
WirePlumber
        ↓
AuraBridge Safe Sink, preferred
        ↓
limiter or fixed-gain stage, if verified
        ↓
FiiO KA11 sink

## 9. AirPlay 2 设计

### 9.1 组件

AirPlay 2 使用：

NQPTP
Shairport Sync
PulseAudio backend
pipewire-pulse
PipeWire
WirePlumber
FiiO KA11

### 9.2 Shairport Sync backend 决策

MVP 不使用 native PipeWire backend。

推荐路径：

Shairport Sync PulseAudio backend
        ↓
pipewire-pulse
        ↓
PipeWire
        ↓
AuraBridge Safe Sink, if implemented
        ↓
FiiO KA11 sink

理由：

- PulseAudio backend 更成熟。

- pipewire-pulse 能把 PulseAudio API client 接入 PipeWire media graph。

- 可以避免 native PipeWire backend 在不同发行版和版本上的不确定性。

- 多协议系统比 AirPlay-only 系统更需要统一 sound server。

### 9.3 构建策略

Agent 需要先运行：

./configure --help | grep -i pulse
./configure --help | grep -i airplay

然后选择实际可用的 PulseAudio backend flag。优先目标是：

./configure --sysconfdir=/etc \
  --with-pa \
  --with-soxr \
  --with-avahi \
  --with-ssl=openssl \
  --with-systemd-startup \
  --with-airplay-2

如果 --with-pa 不存在，以 ./configure --help 输出为准，选择 PulseAudio backend 对应选项。

### 9.4 AirPlay 验收

[ ] nqptp.service active
[ ] shairport-sync.service active
[ ] pipewire-pulse active
[ ] iPhone / Mac sees "Aura Studio 3 AirPlay"
[ ] Audio routes to KA11 through PipeWire
[ ] No ALSA device busy issue
[ ] Safe volume initialization works
[ ] No dangerous initial output level

## 10. Spotify Connect 设计

Spotify Connect 使用 librespot。

推荐路径：

librespot
        ↓
PulseAudio-compatible output
        ↓
pipewire-pulse
        ↓
PipeWire
        ↓
AuraBridge Safe Sink, if implemented
        ↓
FiiO KA11

设备名：

Aura Studio 3 Spotify

验收：

[ ] Spotify app sees the device
[ ] Playback works
[ ] AirPlay and Spotify services coexist
[ ] No ALSA device locking conflict
[ ] Output level is safe

## 11. Bluetooth A2DP 设计

### 11.1 目标

Bluetooth 用于 Android、小米、三星和 PC 的通用兼容播放。

### 11.2 组件

BlueZ
PipeWire Bluetooth support
WirePlumber BlueZ monitor
AuraBridge Safe Sink, if implemented
FiiO KA11 sink

### 11.3 配对策略

蓝牙不能永久 discoverable。

必须提供：

scripts/bt-pairing-window.sh

基础逻辑：

bluetoothctl discoverable on
sleep 120
bluetoothctl discoverable off

### 11.4 抢占风险

蓝牙设备连接后，WirePlumber 默认策略可能自动创建节点并改变路由。2.2 版不假设默认策略一定符合用户体验。

Phase 4 必须做 Bluetooth routing spike：

1. Record PipeWire version.
2. Record WirePlumber version.
3. Identify whether WirePlumber is 0.4.x or 0.5.x+.
4. Use version-matched WirePlumber documentation only.
5. Pair an Android phone.
6. Observe wpctl status before connection.
7. Observe wpctl status after connection.
8. Observe pactl list sink-inputs.
9. Start AirPlay playback, then connect Bluetooth.
10. Start Spotify playback, then connect Bluetooth.
11. Determine whether Bluetooth hijacks routing.
12. If hijack occurs, implement version-specific policy or controller mitigation.

### 11.5 WirePlumber 版本陷阱

必须先检查：

wireplumber --version
wpctl status
pw-cli info all

然后根据版本选择配置方式：

If WirePlumber is 0.4.x:
  Use WirePlumber 0.4 Lua-style configuration and 0.4-era documentation.

If WirePlumber is 0.5.x or newer:
  Use the newer SPA-JSON / JSON-style configuration model.

Do not blindly copy the latest WirePlumber examples.
Do not assume a 0.5.x configuration works on 0.4.x.
Do not assume a 0.4.x Lua rule works on 0.5.x.

### 11.6 可接受的 MVP 策略

如果短期无法优雅处理抢占，MVP 可以采用：

Bluetooth disabled by default
Bluetooth pairing/playback enabled manually
Bluetooth service can be stopped quickly
AirPlay and Spotify remain primary

## 12. DLNA / UPnP 设计

DLNA 是 blocked optional feature，不进核心 MVP。

### 12.1 风险

DLNA 客户端行为不统一，可能出现：

- 连接时强制设置音量为 100%。

- 控制端和 renderer 音量不同步。

- 多个客户端争抢 renderer。

- 输出绕过部分软件音量逻辑。

- 瞬间爆音事件无法用 Bash polling 及时阻止。

### 12.2 2.2 版策略

DLNA is blocked until real-time audio safety is verified.
DLNA must remain disabled by default.
DLNA cannot rely on volume-guard-loop.sh for safety.
Polling-based volume correction is not acceptable as a safety mechanism.

### 12.3 DLNA 解锁条件

只有满足以下条件，才可以启用 DLNA：

[ ] AuraBridge Safe Sink exists, or equivalent protected path exists
[ ] KA11 physical sink is not directly exposed as default sink
[ ] PipeWire-level limiter, fixed-gain filter, or hard cap is verified
[ ] 100% client-side volume command does not create dangerous analog output
[ ] Test performed with Aura Studio 3 physical volume low
[ ] Quick disable script exists
[ ] Agent documents exact renderer and client behavior

### 12.4 候选工具

gmrender-resurrect
Rygel

## 13. 音量安全设计

### 13.1 为什么 KA11 需要更谨慎

KA11 是小尾巴 DAC / headphone amplifier，输出能力比很多普通手机转接线强。它直接接琉璃3 AUX 时，如果音量过高，可能出现非常大的声响。

### 13.2 2.1 版问题

原设想：

volume-guard-loop.sh checks every 5 to 10 seconds.
If volume is higher than 45%, clamp it back.

这个方案有竞态条件。它只能在事故发生后补救，不能阻止瞬间爆音。

### 13.3 2.2 版修正

volume-guard-loop.sh 被降级为：

Recovery tool
Audit tool
Diagnostics tool
Post-failure correction tool

它不是：

Real-time safety mechanism
Limiter
Hard cap
Speaker protection layer

### 13.4 三层音量保护

Layer 1, hardware mixer check：

aplay -l
amixer -c <card_id> scontrols
alsamixer -c <card_id>

如果 KA11 暴露硬件音量控制，设置到安全值。

Layer 2, real-time PipeWire audio safety：

Preferred:
  AuraBridge Safe Sink
  PipeWire filter-chain limiter
  Fixed-gain stage
  KA11 not directly exposed as normal default sink

If this cannot be verified:
  DLNA remains disabled.
  Risky clients remain disabled.

Layer 3, Bash recovery：

safe-volume.sh
volume-guard-loop.sh
systemd timer
logging
recovery only

### 13.5 初始测试建议

Initial PipeWire volume: 20% to 30%
Maximum normal testing volume: 45%
Aura Studio 3 physical volume: low
Phone volume: low
DLNA: disabled
Untrusted clients: disabled

### 13.6 必需脚本

scripts/safe-volume.sh
scripts/volume-guard-loop.sh
scripts/check-ka11.sh
systemd/aurabridge-volume-guard.service
systemd/aurabridge-volume-guard.timer

注意：

The volume guard timer is not a safety guarantee.
It exists for recovery and diagnostics only.

## 14. PipeWire Safe Sink 设计

### 14.1 目标

正常客户端不应该直接把音频送到 KA11 physical sink。推荐建立一个虚拟受控输出：

AuraBridge Safe Sink
        ↓
Limiter or fixed-gain processing
        ↓
FiiO KA11 physical sink

### 14.2 预期优势

- 所有客户端看到的是受控 sink。

- KA11 physical sink 可以隐藏或不设为默认。

- 可以在进入 KA11 前做固定 gain 或 limiter。

- DLNA 这类不可信客户端的输出更容易被约束。

- 后续 source priority controller 更容易实现。

### 14.3 实现要求

Agent 不应假设某个 limiter 插件一定存在。必须先探测系统能力：

pipewire --version
wireplumber --version
pw-cli ls Node
wpctl status
ls /usr/lib/*/ladspa 2>/dev/null || true
ls /usr/lib/*/lv2 2>/dev/null || true

如果可用，可以尝试 PipeWire filter-chain。如果不可用，则记录原因，并保持 DLNA disabled。

### 14.4 Safe Sink 验收

[ ] Clients can output to AuraBridge Safe Sink
[ ] Audio reaches KA11 through controlled path
[ ] KA11 physical sink is not the default sink for normal clients
[ ] Output gain is safe
[ ] 100% client volume does not produce dangerous speaker output
[ ] If not verified, DLNA remains disabled

## 15. Controller 设计

2.2 版不使用 FastAPI 或 Node.js 作为 MVP controller。

MVP controller 采用：

Bash scripts
systemd services
systemd timers
SSH
journalctl logs

原因：

- 更轻量。

- 更适合 headless embedded Linux。

- 更容易 debug。

- 更少依赖。

- 足够完成状态检查、服务重启、蓝牙配对窗口和音量恢复。

后续如果需要 Web UI，可以再用 Python 原生 http.server 或极轻量 CGI 风格实现，不建议 MVP 使用大型框架。

## 16. 推荐 repository 结构

AuraBridge-Pi/
  README.md
  WHITEPAPER_2_2.md
  PROJECT_OVERVIEW_2_2.md
  TROUBLESHOOTING.md

  scripts/
    setup-base.sh
    setup-pipewire.sh
    check-ka11.sh
    safe-volume.sh
    volume-guard-loop.sh
    install-airplay2.sh
    install-spotify.sh
    setup-bluetooth.sh
    bt-pairing-window.sh
    bluetooth-routing-spike.sh
    wireplumber-version-check.sh
    setup-safe-sink.sh
    test-safe-sink.sh
    install-dlna.sh
    status.sh
    logs.sh

  systemd/
    aurabridge-volume-guard.service
    aurabridge-volume-guard.timer
    aurabridge-health.service
    librespot.service
    gmrender.service

  docs/
    hardware.md
    ka11-validation.md
    audio-routing.md
    volume-safety.md
    safe-sink.md
    bluetooth-policy.md
    wireplumber-versioning.md
    airplay2.md
    spotify.md
    dlna.md

## 17. Phase Plan

### Phase 0: 准备阶段

目标：

Prepare hardware, OS image, documentation, and scripts.

检查：

[ ] Raspberry Pi 4 available
[ ] Raspberry Pi OS Lite 64-bit image ready
[ ] microSD card ready
[ ] Pi 4 power supply ready
[ ] USB-A to Type-C female adapter ready
[ ] FiiO KA11 Type-C ready
[ ] 3.5mm AUX cable ready
[ ] Ethernet cable ready if possible

### Phase 1: Base OS + PipeWire + KA11 验收

目标：

Confirm that KA11 works as the primary USB DAC sink.
Confirm PipeWire and WirePlumber versions.
Confirm hardware mixer behavior.
Apply safe initial volume.

必须检查：

lsusb
aplay -l
amixer -c <card_id> scontrols
alsamixer -c <card_id>
pipewire --version
wireplumber --version
wpctl status
pactl list sinks short

验收：

[ ] KA11 appears in lsusb
[ ] KA11 appears in aplay -l
[ ] KA11 appears as PipeWire sink
[ ] WirePlumber version is recorded
[ ] Hardware mixer behavior is documented
[ ] Safe volume command works
[ ] Test audio plays through Aura Studio 3 at low volume

### Phase 2: AirPlay 2

目标：

AirPlay 2 works through Shairport Sync PulseAudio backend and pipewire-pulse.

验收：

[ ] NQPTP active
[ ] Shairport Sync active
[ ] iPhone / Mac sees Aura Studio 3 AirPlay
[ ] Playback works
[ ] PipeWire shows stream
[ ] No ALSA device busy conflict
[ ] Output level is safe

### Phase 3: Spotify Connect

目标：

Spotify Connect works through librespot.

验收：

[ ] Spotify app sees Aura Studio 3 Spotify
[ ] Playback works
[ ] No device locking conflict with AirPlay
[ ] Output level is safe

### Phase 4: Bluetooth A2DP

目标：

Bluetooth works, but does not destroy AirPlay / Spotify experience.

验收：

[ ] Android phone pairs
[ ] Bluetooth playback works
[ ] Pairing window closes automatically
[ ] Auto-connect behavior documented
[ ] Routing hijack behavior documented
[ ] WirePlumber version-specific mitigation implemented if needed

### Phase 5: Safe Sink / Real-time Audio Safety

目标：

Implement or verify a real-time audio safety layer before enabling risky clients.

验收：

[ ] AuraBridge Safe Sink exists, or equivalent protected route exists
[ ] KA11 physical sink is not the default normal client sink
[ ] Limiter, fixed-gain stage, or hard cap is verified
[ ] 100% client-side volume does not create dangerous analog output
[ ] If this cannot be verified, DLNA remains disabled

### Phase 6: Optional DLNA

目标：

DLNA renderer can be manually enabled only after safety testing.

验收：

[ ] DLNA disabled by default
[ ] Manual enable works
[ ] Playback works
[ ] No unsafe volume spike
[ ] Quick disable works
[ ] Behavior documented

### Phase 7: Optional UI / OLED

目标：

Add convenience features after core audio is stable.

Possible features:

Tiny local HTTP status page
OLED display
Physical pairing button
Physical safe-volume reset button

## 18. 验收标准

### Core MVP

[ ] Pi boots headless
[ ] SSH works
[ ] KA11 detected
[ ] PipeWire sees KA11
[ ] WirePlumber version recorded
[ ] Hardware mixer behavior documented
[ ] Safe initial volume applied
[ ] AirPlay 2 works
[ ] Spotify Connect works
[ ] Services survive reboot
[ ] status.sh is useful
[ ] logs.sh is useful

### MVP Plus

[ ] Bluetooth A2DP works
[ ] Bluetooth pairing is controlled
[ ] Bluetooth auto-connect behavior is documented
[ ] Bluetooth does not permanently break AirPlay / Spotify
[ ] WirePlumber version-specific policy is documented if used

### Safe Sink / Audio Safety

[ ] KA11 physical sink is not directly exposed as default sink if Safe Sink is implemented
[ ] AuraBridge Safe Sink works
[ ] Limiter or fixed-gain route is verified
[ ] Polling volume guard is not treated as safety
[ ] DLNA remains disabled if safety layer is not proven

### Optional DLNA

[ ] DLNA works only after real-time audio safety testing
[ ] DLNA disabled by default
[ ] DLNA does not force unsafe volume
[ ] DLNA can be disabled quickly

## 19. 风险矩阵

| Risk | Severity | Probability | Mitigation |
| --- | --- | --- | --- |
| ALSA device locking | High | High | Use PipeWire / pipewire-pulse, avoid direct hw: output |
| DLNA volume spike | High | Medium | Block DLNA until Safe Sink / limiter is verified |
| Bash volume guard race condition | High | High | Treat polling as recovery only, not safety |
| Bluetooth auto-connect interruption | Medium | Medium | Controlled pairing, routing spike, version-specific WirePlumber policy |
| WirePlumber version mismatch | High | Medium | Detect version first, use matching docs |
| KA11 output too loud | High | Medium | Low physical volume, safe initial volume, hardware mixer check |
| USB DAC not detected | High | Medium | check-ka11.sh, validate USB Audio Class behavior |
| Wi-Fi instability | Medium | Medium | Prefer Ethernet or stable 5GHz Wi-Fi |
| Home app instability | Low | Medium | Best-effort only |
| Over-engineered controller | Low | Medium | Use Bash + systemd only in MVP |

## 20. 最终定义

AuraBridge Pi 2.2 是一个以 Raspberry Pi 4 为核心，以 PipeWire / WirePlumber 为音频路由层，以 FiiO KA11 Type-C 为指定 USB DAC 输出设备的多协议家庭音频桥接系统。

它不追求破解小米、三星或 Google 的私有生态，而是通过 AirPlay 2、Spotify Connect、Bluetooth A2DP 和可选 DLNA，把传统 AUX 音箱升级为跨 Apple、Android、Spotify 和局域网媒体生态的现代无线音频终端。

2.2 版的核心工程原则是：

Audio safety must be real-time and inside the audio graph.
Bash polling is not a safety mechanism.
DLNA remains blocked until a real-time limiter or equivalent hard cap is verified.
WirePlumber policy must be version-specific.
FiiO KA11 is powerful enough that output level must be treated carefully.

一句话定义：

AuraBridge Pi 2.2 turns a traditional AUX speaker into a practical multi-protocol network audio endpoint using Raspberry Pi 4, PipeWire, WirePlumber, Shairport Sync, librespot, Bluetooth A2DP, optional DLNA, a real-time audio safety layer, and a FiiO KA11 USB DAC.